DISCONE  v1.0

Mike Davis, AB6SL

03-29-94

DISCONE v1.0 is a quick and dirty but very accurate utility

program that calculates every dimension necessary to construct

a discone antenna.  Discones are famous for their tremendous

bandwidth (SWR of 1.2:1 from 14 to 144 Mhz, for example.)  They

are also infamous for being large, unfortunately.  They have

the low angle of radiation offered by a vertical from the minimum

frequency of operation through about 4 times that frequency, after

which the angle of radiation increases out to the maximum usable

frequency (10 times the minimum.)  They have the gain of a dipole

without the nulls of a dipole.  The current distribution is

greatest at the top of the antenna, unlike a vertical, so they

can be operated much closer to the ground, with almost no ground

losses!  They mate to 50-ohm coax with no feedpoint matching.

If your 80-Meter dipole is suffering ground losses because it's

not 3/4 wavelength high (263-feet !) build a 71-foot tall discone

and enjoy a 360 degree azimuth plot with a low angle of radiation.

Work every freqency from 3.5MHz to 35MHz with 1.2:1 SWR and little

ground loss!

71 feet too tall?  OK, how about 35 feet for 40-Meters through 6?

Using wire radials to make the skeleton, these antennas are cheap!

Given the desired minimum frequency of operation and optionally,

a user-specified critical dimension, Cmin, this program will

calculate all the dimensions necessary to construct a discone

antenna.  The "ARRL Antenna Book" 16th Ed., Chapter 7, pp. 17-20

is required reading!  All formulae were derived from this source.

Construction methods and graphic figures provided there,

especially figure 33 on p. 7-18 make this program's output of

value instead of just confusing.

Antennae constructed using this program will exhibit the optimum

60 degree discone parameters: S=0.3Cmin, D=0.7Cmax, and Ls/Cmin>22.

Bandwidth with a 1.2:1 SWR will be 10:1 !

Try running the program with the same design criteria used for

the 7 to 70 MHz discone documented in the article.  With an Fmin

of 7.0 MHz, a Cmin of 13.5 inches, 8 disc radials and 24 cone

radials, this program spits out the same dimensions derived the

hard way in the article.  It's accurate and you can trust it.

This program assumes a skeleton discone will be constructed but,

works just as well for solid surface discones... just ignore all

input and output regarding "radials."  All relevant dimensions

will be correct.  It is inadvisable to reduce the number of

disc radials to less than 4 or cone radials to less than 8.

The ARRL article indicates a 0.02 wavelength at Fcutoff as a guide

for spacing the radials at the base of a cone.  No matter what

frequency you design for, this works out to 40 cone radials!

The article overlooks a contradicting example they give of an HF

discone with only 24 radials in the cone (8 in the disc)!

Extensive testing was done by WA4RHT, D. Wilson Cooke, with

skeleton discones, as published in the "ARRL Antenna Compendium,

Volume 3" pp. 140 to 143.  He concluded that more spokes are better

but, 8 in the cone and 4 in the disc is as few as you can have

without having to increase the Ls (Slant Height of the cone) to

1/3 wavelength at Fcutoff instead of 1/4 wavelength.

Most commercially manufactured VHF/UHF discones have no more than

8 cone radials and 4 disc radials, but more cone radials will yield

a flatter SWR curve accross the 10:1 bandwidth of the discone.

To use DISCONE.EXE, just execute it at a DOS prompt.  It accepts no

arguments.  You'll be asked to supply the Fmin (Minimum Frequency

at which you desire to operate with an SWR 1.2:1.)

The SWR rises steeply below this frequency.  For the first run, I

would suggest you provide no value for Cmin, the diameter of the

cone at it's narrowest point beneath the disc.  This is where the

cone radials are attached.  READ THE ARTICLES MENTIONED ABOVE!

By taking the default Cmin the first time you run DISCONE for a

given Fmin, DISCONE will calculate the largest allowable diameter

for Cmin that fits within an important performance optimization:

Ls / Cmin > 22.  READ THE ARTICLES!  After determining the largest

allowable value of Cmin, you can run subsequent calculations with

manually entered lesser values for Cmin that better suit your

construction needs.  Remember, smaller Cmin means better bandwidth!

Changing the number of radials from the defaults of 4 disc and 8

cone radials will only change some of the reported values that are

provided for ease of construction.  Calculated values of Fcutoff,

the 4:1 and 10:1 multipliers of Fmin, etc. are not affected by

changing the number of radials from the defaults.  Remember

that you are venturing into inaccurate territory if you specify

fewer than the default.  The formulae used here assume the defaults

to be the minimum number of radials.  No effort is made to calculate

an increased Ls (Slant Height) to compensate for fewer than 8 cone

radials for example.

If a printer is attached, do screen prints to capture results.

Crude, huh?

This program is freeware!  Use it, copy it, distribute it with a

clear conscience.  It's accurate and the price is right.

Enjoy,

Mike Davis, AB6SL