Changes:

CTL-C not really restricted character?

LF not used as alternate EOL

Added info from AA4RE on "M in SID".

...  Hank

BBS Interface Specification

Working Draft

11/13/93

Introduction

Section 1 - Establishing connection

Section 2 - Forwarding

Section 3 - Reverse forwarding

Section 4 - Forwarding to old BBSs

Section 5 - Error Conditions

Section 6 - BNF Summary

Section 7 - Feature Letter Use

Current mailing list

BBS_authors@arasmith.com

bob@arasmith.com (Bob Arasmith, N0ARY)

hank_oredson@mentorg.com (Hank Oredson, W0RLI)

wd6cmu@netcom.com (Eric Williams, WD6CMU)

enge@almaden.ibm.com (Roy Engehausen, AA4RE)

71151.720@CompuServe.COM (Victor Poor, W5SMM)

blloyd@axion.bt.co.uk (Brian Lloyd, G1NNA)

map@hopper.cba.csuohio.edu (Mike Pechura, WA8BXN)

durham@w2xo.pgh.pa.us (James Durham, W2XO)

johan@ece.orst.edu (Johannes Reinalda, WG7J)

trentin@enac.dgac.fr (to J-P Roubelat, F6FBB)

sproul@sproul.com (Mark Sproul, KB2ICI)

mike@km6px.arasmith.com (Mike Stickney, KM6PX)

70007.1365@compuserve.com (Ed Juge, W5TOO)

73126.3260@compuserve.com (Craig McCartney, WA8DRZ)

Via US Mail

Michael Jaggers, WB4TTZ

1377 Columbia Ave, Franklin, TN  37064

Introduction -

This document is meant to define the interaction between two BBSs

exchanging messages. It is not the goal of the document to specify

the interface of the future but merely represent what a BBS can be

expected to see at the current time.

It is hoped that this specification will be followed by another that

defines the base rules for all BBS programs to adopt. This should

eliminate the current problems with incompatibilities and overloaded

operators. If you create a BBS to this specification you should be

able to interconnect with virtually any BBS currently in use. Any

extensions you add will put you at risk, the subsequent spec is to

identify these areas.

In brief the following sequence of events takes place when the BBSs

connect for forwarding:

Master System 			Slave System

------------				--------------

{Connect}

[SID]

N9ZZZ >

[SID]

>

SB ARES @ ALLCA < W7ZZZ $ARES0108

OK

message subject

message routing headers

message body

/EX

>

SB WANT @ ALLUS < W8AAA $1029_N0XYZ

NO

>

F>

SP WA2ABC @ N2AAA < N9AAA

OK

message subject

message routing headers

message body

/EX

F>

{Disconnect}

This represents a simple example of connecting, establishing a common

protocol, forwarding and reverse forwarding. This document will look

more closely at each of these phases.

In the text the BBS initiating the connect will be referred to as the

MASTER and the remote BBS will be referred to as the SLAVE.

Each command and exchange will be presented in BNF (Backus-Naur Form)

which is a context-free grammar widely used to describe computer

language syntax.

All commands and fields are case insensitive.

Meta-syntactic information

[x]   =    Optional: zero or one x

(x y) =    Grouping: treat multiple tokens as a single token

<>	  =    Set

Characters:

NUL =	<0x00>

CNTRL_C =	<0x03>

TAB =	<0x09>

CR =	<0x0D>

LF =	<0x0A>

CNTRL_Z =	<0x1A>

SP =	<0x20>

Character sets:

ALPHA =	<'A'-'Z'>

DIGIT =	<'0'-'9'>

ALNUM =	ALPHA | DIGIT

ASCII =	<0x20-0xFF>

SPACE =	SP | TAB

8BIT  =	<0x01 - 0xff, except CR, LF, CTRL_Z>

Character sequences:

EOL =	CR | CR LF

NUMBER =	DIGIT | NUMBER DIGIT

WHITE =	SPACE | WHITE SPACE

ASCII_STR = ASCII | ASCII_STR ASCII

ALNUM_STR = ALNUM | ALNUM_STR ALNUM

8BIT_STR  = 8BIT  | 8BIT_STR  8BIT

Throughout this document we will present a verbose translation of the

grammar highlighting any points that may not be obvious but could

have significant affect. There will also be examples illustrating what

can actually be expected.

Section 1 - Establishing connection

When connecting to a BBS the first line the BBS sends must be the

SID (System IDentifier). The SID is used to determine what features

are supported by BBS. The existence of the SID implies that the system

supports reverse forwarding and OK/NO message rejection. There still

exist old BBS codes which do not present a SID at connect. Refer to

Section 4 for information on forwarding to these old style BBS sytems.

SID sent by both MASTER and SLAVE

AUTHOR_ID	 =  <ASCII_STR, except '[', ']' and '-'>

AUTHOR_DATA  =  <ASCII_STR, except '[' and ']'>

FEATURE	 =  ALPHA [DIGIT]

FEATURE_LIST =  FEATURE | FEATURE_LIST FEATURE

SID 	 =  "[" AUTHOR_ID "-" [AUTHOR_DATA "-"] FEATURE_LIST [$]"]" EOL

The AUTHOR_ID and FEATURE_SET must be present. If the optional

AUTHOR_DATA is present it must be separated from the other two

fields by a '-'. AUTHOR_DATA may contain the '-' character.

The AUTHOR_ID and AUTHOR_DATA are typically a portion of the authors

callsign or BBS name and version number, respectively.

Each FEATURE consists of a letter, denoting the feature and an

optional number, denoting the feature level. The absence of a

version number implies version zero.

If BID's are supported, '$' must be the last symbol in the FEATURE_SET.

The recommended minimal support is BID and Hierarchical location

identifiers. These are necessary for the proper operation of the network.

EXAMPLE

[ZFJ-2.3-H$]

The original prompt received from the SLAVE BBS may contain additional

information intended for users. As soon as the SID is received by the

SLAVE BBS it should respond with an abbreviated PROMPT, with the

ASCII_STR field being empty.

Section 2 - Forwarding

A message can either be rejected or accepted by the SLAVE. There are

many reasons that the SLAVE my choose to reject a message but the most

common will be duplicate bid, indicating the SLAVE already has the message.

Here is an overview of the exchange.

ACCEPT EXCHANGE:

MASTER		SLAVE

------		-----

PROMPT

SEND_CMD

RESPONSE (OK)

MESSAGE

COMMENT

PROMPT

REJECT EXCHANGE:

MASTER		SLAVE

------		-----

PROMPT

SEND_CMD

RESPONSE (NO)

PROMPT

The SLAVE indicates it's prepared to accept a command by issuing a PROMPT.

The PROMPT is also used as an an acknowledgement that the last message

has be received.

The MASTER initiates the message exchange by issuing a a SEND_CMD. This

command contains all the information necessary for the SLAVE to decide

whether to ACCE or REJECT the message. It contains message type,

destination, originator and possibly a bid.

COMMENT =	<ASCII_STR, except '>'>

PROMPT  =	[COMMENT] ">" EOL

TYPE =	'B' | 'P' | 'T'

CALLSIGN =	<ASCII_STR, except '@', '.', max length 6>

LOC =	<ASCII_STR, except '@', '.', max length 6>

HLOC =	LOC | <HLOC "." LOC, max length 31>

BBS =	CALLSIGN ["." HLOC]

TO =	CALLSIGN [WHITE "@" WHITE BBS]

FROM =	CALLSIGN

BID =	<ASCII_STR, max length 12>

SEND_CMD =	"S" TYPE WHITE TO [WHITE "<" WHITE FROM] [WHITE "$" BID]

HLOC is the hierarchical location of the system. It is defined by

the document "INTERNATIONAL ROUTING DESIGNATORS", ARRL Networking

Conference Proceedings xxx.

The TO field commonly takes the following forms:

callsign @ BBS.hloc		; N6ZFJ @ N6QMY.CA.USA.NA

category @ distrib		; WANT @ ALLCA

category @ BBS.hloc		; WANT @ N6QMY.CA.USA.NA

category @ distib.hloc	; INFO @ ALLCA.CA.USA.NA

The first two are seen most often, but the other two forms are seen.

The first two forms are also normally associated with the

message type of PERSONAL and BULLETIN respectively.

White space is required on either side of the '@' and between the

'<' and the senders callsign. There cannot be white space between

the '$' and the BID.

EXAMPLE

SB WANT @ ALLSCV < N6ZFJ $2345_N6QMY

A "BID" (Bulletin IDentifier) is associated with each Bulletin,

and may be associated with any Personal message.

If a BID is not given explicitly when the message is entered into

the system, and the message type requires a BID, one is created

automatically from the message number and callsign of the system

into which the message was initially entered.

NOTE:

It is suggested that a generated BID have the form nnn_BBS,

where nnn is the message number at the originating BBS system,

and BBS is the callsign of the originating BBS system.

COMMON_BID =	MSG_NUM "_" CALLSIGN

BBS codes are free to create whatever form they wish as long as the

length doesn't exceed 12 characters.

Messages to a distribution, whether they are bulletins or personal

messages, must have a BID. Personal messages to a specific user may

carry a BID if the user supplied one, but will not be given a default

BID. NTS message never carry a BID.

The SLAVE BBS determines from the SEND_CMD whether or not to accept

the message. The reasons for rejection is usually that the message

already exists on the SLAVE system (i.e. a duplicate BID), or possibly

a system related problem.

Sent by SLAVE in response to SEND_CMD (or by MASTER if in reverse

forward mode).

RESPONSE =	ACCEPT | REJECT

ACCEPT =	"OK" [ WHITE COMMENT ] EOL

REJECT =	"N" [ "O" ] [ WHITE COMMENT ] EOL

NOTE:

The syntax of responses has grown in a random fashion. Many BBS programs

expect to see the entire words "OK" and "NO" while others simply look

at the first letter. It is suggested that new BBS codes should present the

words "NO" and "OK", but look for only the first letter. Doing so should

ensure compatibility with existing BBS codes.

EXAMPLE

Master:  SB WANT @ ALLCAN < N6ZFJ $4567_N0ARY

Slave:   NO duplicate bid

Slave:   >

Master:  SB WANT @ ALLCAN < N6ZFJ $4567_N0ARY

Slave:   OK #32190

Master:  message body

Master:  /EX

Slave:   Message being held for sysop review

Slave:   >

It is required that the "OK" and "NO" strings be followed by white

space or EOL. Some BBS programs will fail to recognize them otherwise.

EXAMPLE

Master:  SB WANT @ ALLCAN < N6ZFJ $4567_N0ARY

Slave:   NO, already have it

Slave:   >

This would fail with some existing BBS codes due to

the comma following the "NO" string.

If the SLAVE BBS responds with a REJECT condition the message should

not be sent and the MASTER should ignore any response from the SLAVE

until PROMPT is encountered. The MASTER can then submit a new SEND_CMD,

initiate reverse forwarding or disconnect.

Once the SLAVE has issued the ACCEPT response, the MASTER BBS

should immediately send the message subject, routing headers,

message body and EOM. The SLAVE will acknowledge with PROMPT

once it receives an EOM.

Message Routing Header

SUBJECT =  <ASCII_STR, max length 79> EOL

YEAR    =  DIGIT DIGIT

MONTH   =  DIGIT DIGIT

DAY     =  DIGIT DIGIT

HOUR    =  DIGIT DIGIT

MINUTE  =  DIGIT DIGIT

STAMP   =	"R:" YEAR MONTH DAY "/" HOUR MINUTE ["Z"]

MSG_NUM =	<NUMBER, range 1 - 65535 (0xFFFF)>

HEADER1 =	STAMP WHITE MSG_NUM "@" BBS EOL

HEADER2 =	STAMP WHITE "@:" BBS WHITE [ASCII_STR] "#:" MSG_NUM [ASCII_STR]

HEADER  =	HEADER1 | HEADER2

Message structure

HEADERS =	HEADER | HEADERS HEADER

EOM     =	(CNTRL_Z EOL) | (EOL "/EX" EOL)

BODY    =	8BIT_STR | BODY EOL 8BIT_STR

MESSAGE =	SUBJECT [HEADERS] EOL [BODY] EOM

EXAMPLE

Example message

R:930108/1729 @:N6QMY.#NOCAL.CA.USA.NA Fremont #:1114 Z:94536

R:930108/1259 1530@KA6FUB.#NOCAL.CA.USA.NA

R:930107/1638z @:WD6CMU.#NOCAL.CA.USA.NA "OSKbox" #:10576 Z:94805

R:930107/1045 50724@WX3K.#NOCAL.CA.USA.NA

message body will go here, the blank line above separates the

body from the routing headers. When the body is through it should be

terminated as follows:

/EX

NOTE:

Both forms of the header described above are seen, and should be parsed.

In either case there may be comments found at the end of the header line,

or between the BBS and the MSG_NUM entries in HEADER2.

HEADER2 is derived from the header form used in early BBS codes.

it allows addtional information to be passed in the header and parsed

if desired. Some examples of these additional fields seen are:

Z:zipcode

O:orginator

$:bid

C:name_of_sysops_cat

NOTE:

The time stamp in the routing header must be in GMT. Some existing BBS

codes provide the time stamp in other than GMT, and may include a time

zone indicator after the time stamp. This should be considered in the

design of header parsers.

Each system that handles the message places it's routing header

at the top of the message headers before passing the message to

the next BBS. The headers must begin in column 1. A blank line

separates the routing headers from the body of the message.

The last (oldest) routing header shows the BBS where the message

originated. This header is the only source for this information.

The information about the originating BBS is used to create a

return address for reply messages.

Here are two examples of the same header, in the two different formats:

R:930101/0000 1530@KA6FUB.#NOCAL.CA.USA.NA

R:930101/0000 @:KA6FUB.#NOCAL.CA.USA.NA #:1530

These examples show the different syntax for the message number and

originating BBS address. All fields not shown in the examples are

optional at the discretion of the sysop.

Section 3 - Reverse forwarding

Once all messages have either been successfully forwarded from the MASTER

to the SLAVE, or been rejected by the SLAVE, the MASTER may accept

messages from the SLAVE. i.e. the MASTER and SLAVE exchange roles.

This is called "reverse forwarding".

ACCEPT EXCHANGE:

MASTER		SLAVE

------		-----

PROMPT ">"

REV_FWD "F>"

SEND_CMD

RESPONSE "OK"

MESSAGE

COMMENT

REV_FWD "F>"

REJECT EXCHANGE:

MASTER		SLAVE

------		-----

PROMPT ">"

REV_FWD "F>"

SEND_CMD

RESPONSE "NO"

REV_FWD "F>"

The MASTER indicates that it is ready to accept a message by answering

the SLAVE's PROMPT, ">",  with a REV_FWD command, "F>". At that point the

SLAVE issues a SEND_CMD and the process continues as described in

section 2 but with the rolls of MASTER and SLAVE reversed. The only

difference is that the MASTER will always respond with REV_FWD in

place of PROMPT to indicate successful transfer.

REV_FWD =	"F>" EOL

The reverse forwarding phase is optional and is under the control of

the MASTER. It can be terminated by any of the following:

SLAVE disconnects

SLAVE sends anything other than a legal SEND_CMD

MASTER disconnects

MASTER sends anything other than "F>".

Section 4 - Forwarding to old BBSs

If the SLAVE does not return a SID then it is assumed that it will

not support BIDs nor OK/NO message rejection.

The SLAVE indicates it is prepared to accept a command by issuing a

PROMPT. This PROMPT is also an acknowledgement that the last message

has been received.

The MASTER initiates the message exchange by issuing a SEND_CMD with no

HLOC or BID. The SLAVE has no way to reject the message so the MASTER

proceeds to send the message subject and body without expecting

acknowlegement from the SLAVE.

Since the SLAVE assumes a user is entering the message, it will display

prompts to lead the user through the message sending process, asking for

a subject, prompting for the message body, etc. The MASTER should

consider this text to be part of the PROMPT and ignore it.

As in Section 2, the MASTER waits until PROMPT is received. This is the

acknowledgement that the message has been recieved and the next message

can be sent.

When the MASTER has no additional messages to send it disconnects

from the SLAVE. Reverse forwarding is not supported in this situation.

Section 5 - Error Conditions

When an error of any nature is detected by either the MASTER or the SLAVE,

that station will disconnect at once, without issuing a PROMPT.

Section 6 - BNF Summary

Meta-syntactic information.

[x]	     =	Optional Zero or one x.

(x y)	     =	Grouping.  Treat multiple tokens as single token.

<>	     =	Set.

Characters of interest.

TAB	     =	<0x09>

LF	     =	<0x0a>

CR	     =	<0x0d>

CNTRL_Z      =	<0x1a>

SP	     =	<0x20>

Character sets of interest.

ALPHA	     =	<'A' - 'Z', 'a' - 'z'>

DIGIT	     =	<'0' - '9'>

ALNUM	     =	ALPHA | DIGIT

ASCII	     =	<0x20 - 0xff>

SPACE	     =	SP | TAB

8BIT	     =	<0x01 - 0xff, except CR, LF, CTRL_Z>

Character sequences of interest.

EOL	     =	CR | CR LF

NUMBER	     =	DIGIT | NUMBER DIGIT

WHITE	     =	SPACE | WHITE SPACE

ASCII_STR    =	ASCII | ASCII_STR ASCII

ALNUM_STR    =	ALNUM | ALNUM_STR ALNUM

8BIT_STR     =	8BIT  | 8BIT_STR  8BIT

System identifier syntax.

AUTHOR_ID    =	<ASCII_STR, except '[', ']', '-'>

AUTHOR_DATA  =	<ASCII_STR, except '[', ']'>

FEATURE      =	ALPHA [DIGIT]

FEATURE_LIST =	FEATURE | FEATURE_LIST FEATURE

SID	     =	"[" AUTHOR_ID ["-" AUTHOR_DATA] "-" FEATURE_LIST "]" EOL

System Prompt syntax.

COMMENT      =	 <ASCII_STR, except '>'>

PROMPT	     =	 [COMMENT] ">" EOL

Send Command syntax.

TYPE	     =	'B' | 'P' | 'T'

CALLSIGN     =	<ASCII_STR, except '@', '.', max length 6>

LOC	     =	<ASCII_STR, except '@', '.', max length 6>

HLOC	     =	<LOC | HLOC "." LOC, max length 31>

BBS	     =	CALLSIGN ["." HLOC]

TO	     =	CALLSIGN [("@" | WHITE "@" WHITE) BBS]

FROM	     =	CALLSIGN

BID	     =	<ASCII_STR, max length 12>

SEND	     =	"S" TYPE WHITE TO [WHITE "<" WHITE FROM] [WHITE "$" [BID]]

Message syntax.

SUBJECT   =  <ASCII_STR, max length 79> EOL

YEAR	  =  DIGIT DIGIT

MONTH	  =  DIGIT DIGIT

DAY	  =  DIGIT DIGIT

HOUR	  =  DIGIT DIGIT

MINUTE	  =  DIGIT DIGIT

STAMP	  =  "R:" YEAR MONTH DAY "/" HOUR MINUTE

MSG_NUM   =  <NUMBER, range 1 - 65535>

HEADER1   =  STAMP WHITE MSG_NUM "@" BBS EOL

HEADER2   =  STAMP WHITE "@:" BBS WHITE [ASCII_STR] "#:" MSG_NUM [ASCII_STR]

HEADER	  =  HEADER1 | HEADER2

HEADERS   =  HEADER | HEADERS HEADER

BODY	  =  8BIT_STR | BODY EOL 8BIT_STR

EOM	  =  (CTRL_Z EOL) | (EOL "/EX" EOL)

MESSAGE   =  SUBJECT [HEADERS] EOL [BODY] EOM

Standard message identification syntax.

MSG_ID	     =	MSG_NUM "_" CALLSIGN

Section 7 - Feature Letter Use

Feature $:

Supports BIDs.

Feature A:

F6FBB - Acknowledge for personal messages.

Feature B:

F6FBB - Binary messages supported.

Feature C:

Obsolete:  Supports automatic distribution of current date / time.

(W0RLI, K0ZXF)

Feature F:

F6FBB - Batch forwarding protocol.

Feature H:

Supports Hierarchical Location designators.

Feature I:

Some transport mechanisms do not identify the calling station in the

protocol as AX25 does. To handle this situation, a "null command"

is defined as a line beginning with ';'. This allows the sending station

to send callsigns to satisfy the regulations for station identification.

EXAMPLE

; W5SMM de W0RLI

The receiving station must ignore such lines.

Feature L:

G1nna - Compression.

Feature M:

For AA4RE systems:

Basic level ("M0")

MID = message identifier

Every message has a MID.

The MID looks like a "generated" BID (example 12345_AA4RE).

The "M" letter in the SID indicates that a MID is to be transmitted

with messages that have no BID.

Processing:

If a send command is received by a BBS with a MID, the program

should determine if it is a duplicate.  If it is, the program

should respond with the usual "NO" as if it was matching a BID.

Feature R:

AA4RE - Extended reject responses.

OK

Yes, send to me.  Note that no number is required.	This may

help eliminate phantom message numbers.

NO

This is a real dupe

REJECT

I can't handle this

LATER

Send to me later ...

The LATER is useful when you are already receiving a bulletin on one

connect and someone else tries to send you the same bulletin.

REJECTed messages should be called to the attention of the SYSOP

to see why the reject occurs.

The difference between NO and REJECT is whether or not SYSOP attention

is needed.

Abreviations are alloed: 1 Letter only ... O N L R

There must be a blank after the word or letter.  This one is fine:

N Already have it.

This one is not correct:

REJECT, Something wrong

The comma is not allowed.

Feature S:

AA4RE - Extended "S" commands.

Feature X:

W0RLI - Supports ihave/iwant compressed batch forwarding.

A standard for Data compression during the forwarding

of messages in the Amateur Radio BBS Network.

o ihave/iwant variable batch size protocol

o Feature letter X, commands SS, SX, SY.

o lzh compression

----------------------------------------------------------------------------

Command Definitions and BNF description.

----------------------------------------------------------------------------

N	     = INTEGER, size 32 bits.

protocol   = TBD

batch_size = N

file_size  = N

msg_count  = <0 - 25>

id	     = BID | MID CR

ids	     = id | ids id

byte	     = <0x00 - 0xff)

bytes      = byte | bytes byte

ihave = "SX " msg_count [" " protocol] [" " batch_size] CR [ids]

iwant = "SY " msg_count CR [ids]

isend = "SS " file_size CR bytes

For ihave:

Protocol and batch_size not used in initial implementation.

Default protocol is: lzh compressed export/import file.

For iwant:

SY means want only these.

Note that the SS command must terminate with CR, without LF.

RFC-822 header fields used are:

To:

From:

Subject:

Message-ID:

X-msgtype:

X-BID:

----------------------------------------------------------------------------

The basic interchange (example):

----------------------------------------------------------------------------

bn are BIDs (or MID if message has no BID).

m: is master, s: is slave.

m: SX 5

m: b1

m: b2

m: b3

m: b4

m: b5

s: SY 3

s: b1

s: b3

s: b5

m: SS 26545

m: <sends 26545 byte file>

In the case of any error, the station noticing the error must disconnect.

----------------------------------------------------------------------------

With more detail of one implementation (MB/SERVER processing)

----------------------------------------------------------------------------

Maximum batch size defined globally (default), and per link.

Ability to turn compression on/off (add/drop feature X in SID).

From sending side:

MB:

Scan messages to find a batch just larger than max batch size.

Keep record numbers and cc: id in list.

Send ihave.

Receive iwant.

Mark all messages in ihave and not in iwant as forwarded (dup bid).

Pass iwant list to SERVER in file.

Wait for server acknowledge.

SERVER:

Create export file.

Compress export file.

Signal MB that compressed file ready to send.

MB:

Send isend, send the file.

Receive prompt.

Mark all messages in iwant as forwarded.

From receiving side:

MB:

Receive ihave.

Check bids in ihave.

For all non-dup bids, make iwant list.

Send iwant.

Receive compressed file.

Signal SERVER import file available.

Send prompt.

SERVER:

Decompress file, import messages.

Section 8 - Import / Export files.

Import/Export files may contain any number of messages.

Each message is terminated by "/EX" EOL

RFC-822 header use in import/export files.

The standard RFC-822 keywords which must be supported are:

To:

From:

Subject:

Message-ID:

cc:

Extension keywords which must be supported are:

X-msgtype:	   (W0RLI)

X-BBS-Msg-Type:  (JNOS)

X-BID:	   (W0RLI)

X-BBS-Hold:	   (JNOS)

X-Forwarded-To:  (JNOS)

END OF SPEC