ANTPLOT.

SIDE-TOWER MOUNTED ANTENNA

PATTERN PREDICTION PROGRAM VER. 5.3

By Ronald J. Jakubowski and Alex F. Dolgosh

Copyright 1992-93 The Antenna Specialists Co.

All Rights Reserved

Program No. 3 of the Antenna Specialists RF TOOLS Series

INSTALLATION:

The diskette supplied contains the files necessary to run the

ANTPLOT program.  It may be run from the floppy, or may be copied

into a subdirectory on your hard disk drive (C:\ANTPLOT, for

example).  To run the program, type ANTPLOT and press ENTER.

BACKGROUND:

For years, antenna manufacturers and consultants have known that

antennas mounted off the side of towers exhibit radiation patterns

which are affected by the tower structure acting as a complex

passive reflector.  An "omnidirectional" antenna no longer creates

an omnidirectional field.

In 1969, the engineers at The Antenna Specialists Company mounted

a tower on a rotatable platform on their test range and measured

side-mounted antenna patterns by rotating the entire tower.  As

more and more patterns were measured at different spacings on

towers with different face dimensions, it finally was concluded

that reasonable accuracy could only be obtained by modelling an

exact installation.  The many variables prevented interpolating

results for an installation falling "between" two measured

patterns.

The work done in the 1969-1970 period was archived for possible

future reference.

As time progressed, computers integrated their way into all facets

of business.  The need to provide more detailed systems information

grew, and the ability of systems engineers to perform was greatly

tested.  Fifteen years after the test range data was archived, the

A/S Systems Engineering Department was tasked with the goal of

writing software to predict these patterns.  The result is ANTPLOT

- a program which, when compared to the real data on file for over

two decades, will provide pattern predictions with reasonable

accuracy.  Since many other variables can affect the patterns, use

of this data for system design is at your own risk.

SYSTEM REQUIREMENTS:

ANTPLOT will run on most IBM-compatible PCs.

A configuration utility will appear the first time the program is

run which allows ANTPLOT to be configured for output devices for

printing or plotting.  It may be recalled from the opening screen

for changes.

The plotting routines are written in HP Graphics Language (HP GL)

and the program will drive either an HP 7475A desktop plotter or a

laser printer that is HP GL compatible, such as the HP LaserJet

IIIp.  If a LaserJet IIIp is used, it should have 5 megabytes of

RAM memory, and the "Page Protection" item of the Configuration

Menu should be set to ON to allow complex graphics.  See the HP

LaserJet IIIp Printer Getting Started Guide.

Memory usage is not intensive, so minimal 640K machines have no

problem with ANTPLOT.

Because the program draws screen displays and calculates patterns

point-by-point around the azimuth, a math coprocessor is highly

recommended for speedy execution of the graphic display.

PROGRAM OPERATION:

ANTPLOT is essentially user friendly to the Systems Engineer and

most potential users who have a knowledge of antennas and towers

adequate to interpret the pattern generated by the program.

ANTPLOT prompts the user for the input of several variables - tower

shape, geometry of mounting, operating frequency, face dimension of

tower, spacing of antenna from tower and azimuthal location of side

mount.

As an alternative to spacing, the user is prompted to enter 0

(zero) to receive a recommendation where near omnidirectionality is

desired.  The program will then calculate the pattern for 0.5 ft.

increments out to 8 feet.  A table of ratios of maximum to minimum

signal levels - compared to depth and location of greatest null -

will allow an experienced system designer to select a most

advantageous spacing, with a ratio of 1:1 being omnidirectional.

A pattern may be plotted for any set of variables to study each in

detail.

The menu bar at the bottom of the screen provides options for

output of data.

RFTOOL3A

October 14, 1993