______________________________________________________________________

Section B : configuration verbs : modem commands                    Section C : configuration verbs : system parameters                 Section D : other configuration verbs                               Section E : script commands                                         Section F : script- and event names                                 Section G : Executing special function via interrupt 14h            Section H : the file format used to store voice data

Each file consists of a 16 byte file header, followed by the voice

data.  There's a slight difference between voice files  used  with

ZyXEL modems and voice files for use with other modems (the latter

will be used if "Rockwell" is used in the configuration file).

--

> ZyXEL modems                                                       <

Byte      header contents

00h-04h   "ZyXEL" (ASCII string)

05h       2

06h-09h   0

0Ah       (bits/sample)-1; 0=CELP,1=ADPCM2,2=ADPCM3,3=ADPCM4.

bit 2 should be set if the voice data contains resync

information : 5=ADPCM2, 6=ADPCM3, 7=ADPCM4.

0Bh-0Fh   0

If  bit  2 of byte 0Ah is set, the file might contain resync codes

(two bytes: 10h followed by 12h),  and the data byte 10h is stored

as two bytes, 10h twice.

Normally, bit 2 of byte 0Ah  is  cleared  (this  is  the  standard

"ZFax"  file  format),  and  the data byte 10h will be stored as a

single byte.  When replaying this kind  of file, the 10h has to be

sent twice to the modem.

Except for resync enhancements, this format is fully compatible to

the file format used for  example  by  ZyXEL's  ZFax  and  similar

programs.   The  enhancement  is  known  to  ZyXEL  and  has  been

confirmed  to be used in future versions of ZFax, but until today,

no program except for ZuTSR  and  VoiceConnect is known to utilize

this file format.

> Files recorded with bit 2 set in byte 0Ah therefore cannot be used <

with other programs than those which are  especially  said  to  be

able to utilize this file format.

The  same  applies  to converting those files to Creative Labs VOC

file format: Only ADPCM files with  bit  2 cleared in byte 0Ah can

be converted to VOC using standard  conversion  programs  such  as

VCNVT (by ZyXEL) or ZyXELVOC (by Norbert Igl).

Only  VCNVTA  written  by  Thomas  Haukap is able to convert files

recorded using the enhanced file format to other file formats.

--

> modems equipped with Rockwell chipset                              <

byte      header contents

00h-04h   "ADPCM" (ASCII string)

05h       2

06h-09h   0

0Ah       (bits/sample)-2; also 0=ADPCM2,1=ADPCM3,2=ADPCM4

0Bh-0Fh   0

The data byte 10h will be stored as a single byte.  When replaying

this kind of file, the byte 10h has to be sent twice to the modem.

This file format has  been  derived  from  the ZFax file format as

described  above,  and  currently  is  used  only  by  ZuTSR   and

VoiceConnect.

To  convert ADPCM files of this kind to Creative Lab's VOC format,

at this time there's only Thomas Haukap's VCNVTA program, starting

with version number 1.52.

Hint: Sources for VoiceConnect and VCNVTA can be found in the file

QUESTION.DOC.