� frequently asked questions and the appropriate answers

______________________________________________________________________

� Section A : Who helps, if not this text ?                          �

Questions and  suggestions,  equal  whether  you  are  owner  of a

license for use or not,  are  welcome  in  the  author's  mailbox.

Please  understand,  that  we do not answer questions for which an

answer could be found in this documentation, and which do not need

a deep knowledge of serial communications in detail.

A very important entreaty  :  please  remove all comment text from

configuration files, if you send them to the author together  with

a request for help.

Use  e-mail  or the BBS system of the author for asking questions.

Fax  or  snail  mail  is  answered  only  if  you  give  an e-mail

destination for the answer, for cost reasons.

The author will set up a mailing list server, to send mails  about

ZuTSR news (new versions, etc) to all users who request to receive

this information.  Interested users, please ask for it.

Registered users, who

- know  about  the  basic  principles  of  PCs,  MS-DOS,  resident

programs, mailers and ZuTSR, and who

- like to trial, test,

- who show creativity while searching for possible errors and bugs

and while inventing new features

- who are reachable via e-mail in the international networks by

the author (see list below)

will  be  of  great  help for the further development of ZuTSR.  A

special form  has  not  to  be  filled  out.   The  access  to the

so-called  "ZuTSR  test  file  area"  in  the  author's   mailbox,

containing alpha- and beta- releases, will be given on request.

You might reach the author as follows : (subject to change)

Kolja Waschk                FidoNet  :            2:240/5814

Oevelgoenne 23              ZyXELNet :           16:100/2215

22605 Hamburg               UniNet   :           127:100/906

Germany                     UseNet   : kawk@zerebrm.hanse.de

Phone   (11:00 - 22:00 GMT+1 resp. GMT+2 in summer (CET/CEST)),

BBS     (7:15 - 1:15, requests 24h except 02:30-03:30 GMT)

Fax :

+49-40-8804056

--

> Important ! Commercial enquiries,  product orders, etc. must not   <

be  sent  through systems of the above listed electronic networks,

as these nets are meant  only for private, non-commercial mailing.

Please send those mails always direct, as a "crashmail",  fax,  or

as a normal letter !

You  may  request  the  most recent public version of ZuTSR in the

author's BBS at any time, using the magic name

ZUTSR    - (ca.  60k) program & english documentation

ZUTSRG   - (ca.  40k) german documentation, without program

ZUTSBETA - small updates without complete documentation, mostly

prereleases

______________________________________________________________________

� Section B : frequently asked questions                             �

--

> After ZuTSR  is  loaded,  the  mailer  does  not  answer any call. <

Instead the message "NO CARRIER" appears a few seconds  after  the

first ring signal.

This  happens,  because ZuTSR might try to send commands faster to

the modem than that is able  to receive them.  Increase the values

for "CommandDelay", "CharDelay", and/or insert delays in  critical

modem commands (~D, ~P, ~p).  To start, set

CommandDelay 200

CharDelay 20

Please read the following notice also.

--

> If  the  option  'Debug' is enabled, everything works wonderfully. <

But as soon as  it  is  taken  from  the config, messages won't be

replayed anymore and other.

This points to a timing problem also.  Try to find  out  which  is

the  last  command  sent to the modem before the error occurs, and

then add a leading or trailing delay in the command (~P, ~p, ~D)

--

> After ZuTSR detects a  human  call,  an external program should be <

executed to handle the voice call.  The external program  actually

starts  up,  but  before  it does anything, something hangs up the

line...

First, determine where the line gets  hung up.  There are at least

four different possibilities:

(a) ZuTSR itself hangs up (see next question & answer)

(b) The mailer hangs up, before it  shells  out  to  the  external

program.   Normally, this means that your mailer is configured

to "drop DTR", or,  more  generally,  to initialize the modem,

before it executes the external program.  Please refer to  the

documentation of your mailer to solve this problem.

(c) Somewhere  in  your  batch file (if used)  the connection gets

terminated.  Check your batch program.

(d) The   external  program   generally  terminates  any  possibly

existing connection before starting up.  Look  for  a  command

line  switch  or  anything  similar for starting the software,

that would disable this behaviour.

Whether you have problems with  (a)/(b) or (c)/(d) could be simply

tested; just replace the execution of the external  program  by  a

small delay or anything similar.

--

> ZuTSR immediately hangs up after handling an incoming call, and no <

connect  message  is  simulated; instead, I get "NO CARRIER" in my

mailer's log file.

Make sure that you have the script command "S" as the last command

of the appropriate script.   This  is  NOT  the case in the sample

script included with the sample configuration files.  Please refer

to the samples in INSTALL.DOC and the informations in REFER.DOC to

learn about the meaning of the script commands.

--

> ZuTSR does NOT hang up the modem after handling a  call;  instead, <

the  mailer  just waits for the next call on a line which is still

switched off hook.

When simulating a connect  message,  ZuTSR  does not actually make

the modem go on hook, to enable external programs to take  control

over  the  call.   Either  disable  the  simulation of the connect

message (remove  the  script  command  "S"  from  the  end  of the

appropriate script, or replace it by "H"), or  let  some  external

program  do the hang up (for example, by using ECHO ATH > COM1, if

your modem is connected to Com1)

--

> Voice calls are passed correctly to an external program, but ZuTSR <

gets restarted by the  external  program  and  tries to handle the

call again (as from the beginning) ?

If you've configured the external program to use the Fossil driver

for data transfer to the modem, all data will pass ZuTSR also.  To

avoid ZuTSR from reacting on another "ATA", sent by  the  external

program for example to establish a data connection, simply disable

ZuTSR  before  executing  the  external  program  (ZUTSR  /D)  and

re-enable  it  afterwards  (ZUTSR /E).  Another option would be to

configure the external program  not  to  use the Fossil driver, if

available.

--

> Voice files cannot be replayed using ZFax; there's only noise.     <

Also, converting the files to another file format using  VCNVT  or

similar does not work.

Probably you've set the parameter "Compression" to 6,7 or 8. Using

this setting, ZuTSR will generate voice files, which are not fully

compatible  to  the  file format used by ZFax.  Files of this type

can be replayed only with  few  other programs than ZuTSR.  Please

see also the informations about file formats in REFER.DOC.

Other reasons for noise might be slow hardware, or a locked serial

speed below 38400 bps.  Often, you might experience  this  problem

when  running ZuTSR in (pseudo-) multitasking environments like MS

Windows, or under OS/2,  where  data gets lost easily.  Hopefully,

the option "WindowsBlock" might help you in the latter situation.

--

> Sometimes, when a single beep tone should be generated, TWO beeps  <

can be heard; sometimes,  ZuTSR  seems  to  halt  completely,  and

sometimes, a beep tone does not stop anymore at all...

This  behaviour  is  a bug in many modems equipped with a Rockwell

chipset, using old firmware.  The only  thing  to do for you is to

look for a newer firmware or to replace the beep tone by a  signal

tone sampled as a voice file.

--

> ZuTSR halts while doing the script command "Q".                    <

To  implement  this action on Rockwell based modems as on other IS

101 compatible modems, ZuTSR  uses  a  method that requires a very

stable and well-programmed firmware in the modem, that is able  to

switch  between  recording  and  replaying  mode  very quickly and

reliable.  Some modems show their limits to you here.

--

> I'm using a ZyXEL modem, but cannot utilize the compression modes  <

4 and 8.

1. 4bits/Sample  Z-ADPCM  is  available  starting  with  6.22;  no

earlier firmware version is capable of this.

2. The serial link  speed between modem and computer has to be  at

least  set  to 57600 bps (a Fossil driver setting).  This speed

will definitely  cause  problems  on  slow  systems, ie.  those

which are not equipped with a 16550 chip, or similar.

Yet, simply use another compression mode.

--

> Regardless whether I'm using the  script  commands  "r"  and  "P", <

ZuTSR always will store the data in "Destination A".

This  is  a  limitation  of  the  unregistered  version.   If this

behaviour remains even after you've  inserted  a valid key code in

your configuration, please contact the author.

--

> What can I  do  to  force  ZuTSR  to  hang  up  immediately  while <

answering  a  call,  to  take over using my phone connected at the

rear of the modem or parallel to it ?

Press the <Esc> key  on  the  keyboard  of  the  PC where ZuTSR is

running on.

--

> I'm using a ZyXEL modem, and therefore cannot make ZuTSR to detect <

automatically when I've taken the phone set connected to  it.   It

will  not interrupt the call handling, and I have to stand up from

bed and make many steps to  reach  the PC, to press the <Esc> key,

before I can talk in the phone...

Use a micro switch located under the phone hand set,  and  connect

it  to  some  unused digital input at your PC (for example, to the

game port instead of a  joystick  button, or to the printer port).

Then, the configuration verb "HardBreak" can be used to make ZuTSR

react on a release of this micro switch similar to the pressing of

the <Esc> key on the keyboard.  Hint: Use a switch that is  opened

normally,  and closed when you lift up the phone; otherwise, ZuTSR

will  not  operate  properly  when  the  switch  is  not connected

temporarily.

--

> After a call has been assumed to be  a  human  call,  ZuTSR  shall <

record  a  message.  It does so, but the recording will be stopped

too early, interrupting the caller while speaking.

ZuTSR is very sensitive  even  for  very short periods of silence.

To avoid this, please read the answer on the next topic.  The fact

that ZuTSR obviously stops the  recording  even  if  there  is  no

silence,  but  while  the caller is actually talking, is caused by

another  problem.   ZuTSR's  internal  audio  signal  analyzer  is

optimized to distinguish between  human  and machine calls, not to

handle human calls.  If tends to be a real  problem,  disable  the

internal  analyzer  (remove "IntAnalyze" from your configuration),

or use external programs for the voice call handling.

--

> I want  to  change  the  silence detection interval (configuration <

verb "Silence") between first and following recordings.

Only if you're NOT using "IntAnalyze", you may do this by using  a

simple  trick: Just let a modem command, which is used only before

starting another recording, change the silence detection interval.

For example, add the setting's  change  to the "Beep" command, and

let ZuTSR send it to the modem before  recording  (script  command

"B").

--

> Are there any other programs to utilize the voice mode, except for <

ZuTSR ?

Beside  some  programs,  that  can  be  bought  only directly from

commercial distributors, there are some  very good (if not better)

programs available as Shareware or in the  Public  Domain,  to  be

obtained  from  many  public  BBS  systems,  as  from the author's

system.

The following lists only a few of them:

ZUtil - written by the same author as ZuTSR

ZUtil is a program to be used on the DOS command line, or within

batch programs.  It enables you  to record voice, send voice, to

send commands to the modem and await its  responses,  and  more.

Using  this  program,  and having little knowledge of using DOS'

batch program language,  you'll  be  able  to  build up your own

powerful voice  call  handling  system  to  answer  calls,  make

outgoing calls, and more.

The  current version of ZUtil is quite old, and can be used with

ZyXEL modems only yet.  Planned  for the future is a combination

of ZuTSR and ZUtil, which will work with other  modem  types  as

well.

VoiceConnect - by Thomas Haukap, Haukap Kommunikationstechnik

VoiceConnect  obviously  is  the  most  powerful  voice  program

available  on  the  shareware market yet; beside many voice call

handling  options  it   features   an   internal  fax  receiver,

transmitter and viewer.  Also available are features to  provide

fax  documents  on demand, or to be polled with any fax machine.

It is a very big  program, nevertheless it's running more stable

than many other commercial programs.  Not to forget, it is  able

to  utilize  a  SoundBlaster(tm) compatible sound card to replay

and  record  voice  files,  and  to  listen  to  incoming calls.

VoiceConnect can be used with almost any modem that has a  voice

mode,  ZyXEL  modems,  Rockwell  based  modems, and other IS 101

compatible modems.  Currently, VoiceConnect  is the only program

beside ZuTSR which is able to record and  replay  files  in  the

enhanced  ZFax  file  format,  to  store  Rockwell  data  and/or

Resync-data.

A port to MS Windows will be available in the late summer of '94.

VoiceConnect can be copied from Thomas Haukap's BBS, the

Intuition : +49-40-8705378.

VCNVTA - by Thomas Haukap, Haukap Kommunikationstechnik

VCNVTA is a little utility to  convert voice files from one file

format (ZyXEL CELP, Z-ADPCM, R-ADPCM) to Creative Lab's VOC file

format, and vice versa.  VCNVTA is also  capable  of  converting

files recorded with ZuTSR in the compression modes 6,7 and 8.

VCNVTA can be copied from Thomas Haukap's BBS, the

Intuition : +49-40-8705378.