��  � ���Ŀ ���Ŀ ���Ŀ ����Ŀ ���Ŀ ����Ŀ ���Ŀ

��  � ��    ��  � ��      ��   ��  �   ��   ��         Version 4.15

��  � ���¿ ����� ���¿   ��   ���Ĵ   ��   ���¿

��  �    �� �� �     ��   ��   ��  �   ��      ��         4/27/96

����� ����� �� �� �����   ��   ��  �   ��   �����

Interpreting the USRSTATS Report

�����������������������������������������������������������������������

Public posting of this document in its unmodified form is permitted.

�����������������������������������������������������������������������

The USRSTATS report is divided into several sections, like this:

���������������������������������Ŀ

�             Info                �

���������������������������������Ĵ

�               �                 �

� Error Control � Data Pump stats �

� and DTE stats �                 �

�               �                 �

�����������������������������������

�����������������������������Ŀ

�                             �

�        Channel Probe        �

�    (V.34 and V.FC modes)    �

�                             �

�������������������������������

�����������������������������Ŀ

�                             �

�       Current settings      �

�          (optional)         �

�                             �

�������������������������������

Error Control and DTE stats - This section contains information about

the Error Control and Data Compression protocols currently active.

This includes MNP, LAPM, V.42, and V.42bis.  It also displays the

number of characters sent and received by the attached terminal.

Data Pump stats - This section contains information on the operation of

the Data Pump.   Specifically, it provides details of the modulation

protocol that was negotiated, the impairments that are affecting the

connection, and how the modem has responded to those impairments.

Channel Probe - This section is applicable only to connections made

under V.34 and VFC modulation.  It shows details of the frequency

response and bandwidth characteristics of the physical link.  (Phone

line.)

Info - This field contains the USRSTATS banner, and information used

for record-keeping purposes, such as the date and time, or the name of

the system being called.

�����������������������������������������������������������������������

Here's a sample of a V.34 connection report:

��������������������������������������������������������������������Ŀ

� USRSTATS Version 4.15 � 04-27-96  11:35 � Caller 0                 �

��������������������������������������������������������������������Ĵ

� Elapsed Time           00:00:18 � Modulation                 V.34+ �

� Blocks Received              15 � Speed                31200/28800 �

� Blers                         0 � Symbol Rate            3429/3429 �

� Blocks sent                   5 � Carrier Frequency      1959/1959 �

� Link Naks                     0 � Trellis Code       64S-4D/64S-4D �

� Blocks resent                 0 � Nonlinear Encoding         ON/ON �

� Link Timeouts                 0 � Precoding                 OFF/ON �

� Chars sent                    0 � Shaping                    ON/ON �

� Octets sent                  10 � Preemphasis                  4/6 �

� Chars lost                    0 � Rx Lev/TX Lev/SNR 25.2/12.0/36.8 �

� Chars Received                0 � Echo Loss   Near 17.0   Far 40.0 �

� Octets Received             239 � Roundtrip Delay                4 �

� Protocol              LAPM SREJ � Retrains Request/Grant       0/0 �

� Block Size                  244 � Fallback                 Enabled �

� Window Size                  15 � HST Line Reversals             0 �

� Compression              V42BIS � HST Equalization            Long �

� Dictionary Size            2048 � SV: 04/15/96       DSP: 04/11/96 �

� String Length                32 � Reason:                   Online �

����������������������������������������������������������������������

���������������������������������������������������������������Ŀ

� -22 � � � � � � � � � � � � � � � � � � � � � � � � � � �   0 �

� -24 � � � � � � � � � � � � � � � � � � � � � � � � � � �   2 �

� -26 � � � � � � � � � � � � � � � � � � � � � � � � � � �   4 �

� -28 � � � � � � � � � � � � � � � � � � � � � � � � � � �   6 �

� -30 � � � � � � � � � � � � � � � � � � � � � � � � � � �   8 �

� -32 � � � � � � � � � � � � � � � � � � � � � � � � � � �  10 �

� -34 � � � � � � � � � � � � � � � � � � � � � � � � � � �  12 �

� -36 � � � � � � � � � � � � � � � � � � � � � � � � � � �  14 �

� -38 � � � � � � � � � � � � � � � � � � � � � � � � � � �  16 �

� -40 � � � � � � � � � � � � � � � � � � � � � � � � � � �  18 �

� -42 � � � � � � � � � � � � � � � � � � � � � � � � � � �  20 �

�Level�����������������������������������������������������Atten�

�       0 0 0 0 0 0 1 1 1 1 1 1 1 2 2 2 2 2 2 3 3 3 3 3 3       �

�       1 3 4 6 7 9 0 2 3 5 6 8 9 1 2 4 5 7 8 0 1 3 4 6 7       �

�       5 0 5 0 5 0 5 0 5 0 5 0 5 0 5 0 5 0 5 0 5 0 5 0 5       �

�       0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0       �

�����������������������������������������������������������������

�����������������������������������������������������������������������

Error Control Group:

Blocks Sent/Received - The number of Blocks sent and received by the

Error Control protocol.

Blers - (Block Errors)  This field is a count of errors in received

Error Control Protocol or Data blocks.   These errors are caused by

noise or other impairments in the channel, and small numbers of them

are to be expected.  On severely impaired lines, values in the hundreds

or thousands may be seen.  This field is one of the key indicators of

the receive channel quality.  This number is a count of 50 millisecond

time units during which one or more errors occurred, and is therefore

related to the elapsed time of the connection as well as the number of

blocks received.

Link Naks - This field indicates the number of times the remote modem

requested the re-send of one or more blocks of data.  Since it is

possible for the request to encompass more than 1 block, this number

does not necessarily indicate the actual block count.  The remote

modem requests a re-send when a data block is corrupted by impairments

in the channel.

Blocks resent - This field is a count of transmitted Error Control

protocol blocks that were re-sent at the request of the remote

receiver.  The Link Naks field counts these requests, and is related

to the number of blocks resent.

Link Timeouts - This field indicates the number of times the Error

Control protocol did not receive a response from the remote modem

within the expected time frame.  This is normally caused by channel

impairments, and also occurs when the remote receiver is being flow

controlled by its terminal.

Chars Sent/Received - These fields count the number of characters sent

and received through the modem's DTE interface.   These fields are only

updated periodically when in the on-line state.  After the modem goes

off-line, the true value is shown.

Octets sent/Received - Octets are compressed data units.  The

compression ratio can be determined by dividing Characters by Octets.

Chars lost - This field indicates the number of characters lost in the

Transmit Buffer, due to buffer over-runs.  This field should always be

zero.  Non-zero values indicate a problem with Transmit Data flow

control in the terminal.

Protocol - This field indicates the Error Control protocol in use.

Possible values include NONE, HST, MNP, and LAPM.  (LAPM is sometimes

referred to as V.42, although it is actually only one part of the V.42

spec.)   USR modems will prefer LAPM over MNP, although they can be

forced into MNP mode.   HST Error Control is only used in conjunction

with the proprietary HST modulation mode.

Block Size -  The data frame size used in LAPM and MNP modes.  Larger

block sizes result in less protocol overhead, and faster throughput.

This is a negotiated parameter, and will vary depending on the remote

modems capabilities.

Window Size -  The number of LAPM or MNP data frames that may be in

transit without being accounted for at any moment in time.  Larger

window sizes can improve performance under conditions of high round-

trip delay.   This is a negotiated parameter, and will vary depending

on the remote modems capabilities.

Compression - This field indicates the Data Compression protocol in

use.  Possible values are NONE, MNP5, or V42BIS.

Dictionary Size -  The number of entries in the V42bis compression

table.   Larger numbers indicate greater potential compression

performance with certain data types.  This number may vary when

connecting to modems from different manufacturers.  USR modems default

to a Dictionary Size of 2048 entries.

String Length -  The length of each string in the V42bis compression

table.  Larger numbers indicate greater potential compression

performance with certain data types.  This number may vary when

connecting to modems from different manufacturers.  USR modems default

to a String Length of 32 characters.

�����������������������������������������������������������������������

Data Pump Group:

Modulation -  This field indicates the modulation protocol negotiated

for this connection.  Possible values are V.34, VFC, V.32/bis/terbo,

HST, V.22bis, and several others.  Note that the USR Courier modems

will report V.32/bis/terbo for ANY of the V.32-type modulation schemes.

HST is USR's proprietary High-Speed protocol.

Speed - This field indicates the current bit rate of the connection, in

Bits Per Second (BPS).  For normal V.32/bis/terbo AND VFC modulation, a

single speed will be displayed since those protocols require the

receive and transmit channels to always run at the same speed.  This is

an important point, because it means that both channels are limited to

the speed of the LOWER of the two directions.  USR's ASL feature

enhances the V.32/bis/terbo protocols by allowing the transmit and

receive channel speeds to be adjusted independently, maintaining

maximum throughput in each channel regardless of the "direction" of the

channel impairments.  In this case, two speeds will be displayed, which

are the Receiver and Transmitter speeds, respectively.  Other fields in

the Data Pump Group which display 2 values follow the same convention

of Receiver/Transmitter.

�����������������������������������������������������������������������

This concept of independent channel speed is an integral part of the

V.34 protocol, and is one of the key improvements over VFC.

�����������������������������������������������������������������������

Symbol Rate - This field indicates the Symbol Rate of the receiver and

transmitter, respectively.  A Symbol is a waveform transmitted by the

modem, which contains a certain number of encoded bits of data to be

moved across the link.  The receiving modem decodes this waveform,

recovers the package of bits, and re-assembles it.  (The noise levels

in the channel determine how many bits are encoded in each symbol.

Lower noise levels allow a greater number of bits per symbol.)  The

design of the telephone system limits how many of these symbols may be

sent across a phone line each second.  Symbols cannot be sent faster

than the *bandwidth* available through the phone line.  V.34 and VFC

modulation allow adjusting this symbol rate to any of six possible

values, to obtain the best match with the available bandwidth.  Other

protocols only allow a single, fixed value for the symbol rate,

regardless of the bandwidth of the link.  This field has a very direct

link to the overall connection speed, and under V.34 and VFC

modulation, it is directly related to the available bandwidth as

determined by the line probe.

�����������������������������������������������������������������������

The V.34 protocol allows independent adjustment of the Symbol Rate in

the receive and transmit channels, while VFC requires both channels to

run at the same Symbol Rate, the LOWER of the two.

�����������������������������������������������������������������������

Carrier Freq - This field indicates the Carrier Frequency (in Hz) of

the receiver and transmitter, respectively.  Under V.34 and VFC

modulation, the Carrier Frequency can be one of several values,

determined during the handshake and line probe processes.  Under other

modulation types, the Carrier Frequency is a single fixed value.

Trellis Code - This field indicates the type of Trellis Code in use by

the receiver and transmitter, respectively.  Trellis coding improves

the system's noise immunity.  The type of coding may vary when

connecting modems from different manufacturers.

Nonlinear Encoding - This field applies only under V.34 and VFC

modulation, and shows the state of the received signal and transmitted

signal, respectively.  Nonlinear Encoding is an operation performed on

the transmitted signal to improve the operation of the receiver in the

presence of nonlinear distortion.  The modem will automatically

negotiate this option during the handshake.

Precoding - This field applies only under V.34 and VFC modulation, and

shows the state of the received signal and transmitted signal,

respectively.  Precoding is an operation performed on the transmitted

signal to reduce the effects of noise multiplication in the adaptive

equalization process.  The modem will automatically negotiate this

option during the handshake.

Shaping - This field applies only under V.34 and VFC modulation, and

shows the state of the received signal and transmitted signal,

respectively.  Shaping is an operation performed on the transmitted

signal to improve the operation of the receiver in the presence of

certain types of noise.  The modem will automatically negotiate this

option during the handshake.

Preemphasis Index - This field applies only under V.34 and VFC

modulation, and shows the Index value for the receiver and transmitter,

respectively.  Preemphasis is a way of compensating for poor phone line

bandwidth.  If the line has poor bandwidth, the upper frequencies of

the transmitted signal can be boosted to compensate for the roll-off.

The modem will select 1 of 10 possible values, based on the actual

roll-off rate of the phone line.  This field indicates which value has

been chosen, larger numbers mean that more preemphasis is being applied

to compensate for the line.  The modem will automatically negotiate

this option during the handshake.

Rx/Tx Level - This field indicates the level (in -dB) of the receive

and transmit signals, respectively.   Standard transmit level for

US/Canada modems is approximately -10 dB, although under V.34 and VFC

modulation, the levels are negotiated and adjusted by the modem during

the handshake.  Receive level can vary widely, depending on the

conditions on your local phone line, the line at the remote modem, and

any long-distance or inter-office carrier facilities.   *Typical*

values will range from -40 dB at the low end, to -15 dB at the high

end, with figures in the -20 to -35 range being most common.   Extreme

values in either direction probably indicate a problem in your local

loop, which the phone company may be able to adjust.

SNR (Signal to Noise Ratio) - The number displayed is the ratio of the

Receive Level to the Noise Level.   The Absolute Noise Level (in dBm)

can be computed from the SNR and Receive Level.  In a "noiseless"

channel, the Absolute Noise Level is approximately -60 dBm.   SNR is

one of the most important factors in modem performance.   Lower SNR

values generally result in lower connect speeds.   Note that a poor SNR

does not necessarily mean that the Noise Level is too high; it can also

mean that the RX Level is too low.   Typical SNR values range from 15

to 35 dB.   Larger values indicate better SNR.

Near Echo Loss - Near End echo occurs when the transmitted signal is

reflected from the local modem's hybrid circuit due to impedance

mismatch. The number displayed is the ratio of the Transmit Level to

the Near End Echo Level, or the amount of attenuation the Near End echo

has when compared to the transmitter.   Under normal conditions, the

Near End echo is approximately 30 dB below the TX signal level.  Larger

numbers indicate better impedance matching at the local modem.

Far Echo Loss - Far End echo occurs when the transmitted signal is

reflected from the REMOTE modem's hybrid circuit due to impedance

mismatch.  This echo experiences both the forward channel and reverse

channel losses, and is usually very small in magnitude.  The number

displayed is the ratio of the Receive Level (which has the reverse

channel loss embedded within) to the Far End Echo Level along with the

forward channel loss.   Typical values of Far Echo Loss are

approximately 30 dB.  Larger numbers indicate better impedance matching

at the remote modem.

Roundtrip Delay - This field indicates the amount of time (in

milliseconds) it takes for the modem to hear a reflection of it's own

transmitter.  This is closely related to the physical length and

transmission path of the phone link, and it affects the operation of

the modem's echo canceller.  This delay also affects the timing of the

modem's handshaking routines.

Retrains Requested - This field is a count of the number of times the

modem sent a request for a re-train to the remote end.  This happens

when the line quality degrades to the point where the modems lose sync

with each other.  Note that under some modulation types, and with

certain configuration settings, the remote modem is NOT required to

*honor* this request, it may be simply ignored.  This field corresponds

with the "Retrains Granted" field of the REMOTE modem, NOT the field in

THIS report.

Retrains Granted - This field is a count of the number of times the

modem responded to a re-train request from the remote end.   This field

corresponds to the "Retrains Requested" field of the REMOTE modem, NOT

the field in THIS report.

Fallback - This field indicates whether the modem has performed a speed

shift, either up or down, at any point during the connection.

"Disabled" means that there has been no speed shift, "Enabled" means

there has been a speed shift.

HST Line Reversals - This field is only meaningful for connections

using HST modulation.  It then indicates the number of times the modem

switched the directions of the high-speed forward channel and the low-

speed back channel.

HST Equalization - This field indicates whether the modem is using

extra pre-emphasis on the transmitted signal.  This field applies only

to HST modulation.

SV - This field indicates the revision date of the modem Supervisor

(Controller) firmware.

DSP - This field indicates the revision date of the modem DSP (Data

Pump) firmware.

Reason - This is the "Disconnect Reason" reported by the modem.

�����������������������������������������������������������������������

Line Probe Group:

Line Probing only applies to V.34 and VFC modulation.  This is a test

to determine the actual bandwidth available over the phone line.

During the handshake, the modems send a series of tones to each other,

at known levels and specific frequency points.  The modem calculates

the level of the received signal at each point, and therefore can

determine the maximum bandwidth available for use.  This section of the

report shows each of those frequency points, and, on the left side of

the graph, the signal level (in -dB) at that point.  (Levels are

referenced from 0.)  The right side of the graph shows the attenuation

at each frequency, relative to the HIGHEST level recorded.  (This

corresponds to the dynamic range of the received signal.)   If any of

the signal levels fall below -44 dB, the chart will include a line of

==== characters to show that threshold.  Generally speaking, values

below about -40 dB become difficult for the modem to interpret,

depending on other line conditions.   If the calculated noise level

falls within the range of the chart, a line of **** characters will be

included to mark that level.   The effective bandwidth calculated here

directly affects the Symbol Rate, and overall connect rate.  The modem

will select the fastest Symbol Rate that can be reliably handled by

this bandwidth.

�����������������������������������������������������������������������

Info Group:

This field shows the USRSTATS banner and version number, and the date

and time that the report was created.  (Not when the data was

captured!)  It will also optionally show the Caller and Node number

(when used with the PCBoard PPE), the last number dialed (in stand-

alone mode), the elapsed time of the current call, or other data passed

by a "door" program on a BBS.

�����������������������������������������������������������������������

Settings Group:

This is an optional display of the current modem settings at the time

the report data was captured.

�����������������������������������������������������������������������

[END]