After many gruling hours of trial and error setting combinations and a lot

cooperation from some friends who were patient enough to let me call there

modem all day I've found a string that works for the Reveal 14,400 V32 bis

modem.  This was accomplished playing against a Zoom modem, I haven't tried

it with other brands yet but minor, if any, alterations should do the trick.

*****Simply replace your MODEM.CFG file with the enclosed file.*****

The following is an explanation of the settings:

&F - Reset Factory Default.

\N1 - Direct data-link only.

&D2 - Modem hangs up, returns to the Command Mode after DTR toggle.

&C1 - Turn on Carrier Detect Signal when remote carrier signal

is present.

\G0 - Disable DCE flow control.

\Q0 - Turn off flow control.

B8 - Set Baud rate to 9600.

\J1 - Enable serial port data rate adjustment so serial data rate

automatically adjusts to match the modem-to-modem data rate.

%C0 - Disable V.42bis/MNP Class 5 data compression.