Domain Upload Tester  (c) 1993 Domain Entertainment

Setup/Configuration Documentation

DESCRIPTION/FEATURES

--------------------

Domain Upload Tester (DUTESTER.EXE) is an upload event for use with

Synchronet BBS software.  Dutester has the ability to test the file integrity

of files with the following extensions: GIF, ZIP, ARJ, LZH, ICE, PAK, ARC, ZOO,

SQZ, TD0, EXE, and COM.

Dutester can also scan files, as well as files within archives, for

viruses using one of the following virus scanning programs: SCAN.EXE and

NETSCAN.EXE (McAfee), NAV.EXE (Norton Anti-Virus), VPCSCAN.EXE (Datawatch's

Virex), F-PROT.EXE (Frisk Software), HTSCAN.EXE (Harry Thijssen), and

TBSCAN.EXE (Thunderbyte).

Dutester will check compressed files WITHIN compressed files (any format

including ZIP, ARJ, LZH, ICE, PAK, ARC, ZOO, SQZ) up to 10 levels deep, while

scanning all files for viruses, as well as testing all archives, TD0, GIF, EXE,

and COM files that are within the other levels.  For accuracy, Dutester doesn't

check the file extension to determine the file type, instead it checks the file

signature (so that files which have been renamed with a different extension can

still be properly tested).

Dutester will remove, or optionally replace, any existing comments in

.ZIP, .ARJ, .ZOO, .SQZ, .ARC, and .PAK type files.

Dutester will check for files older than a specified number of days, and

it will check for files in the first level of archives older than a specified

number of days.  In archives, the program compares the number of days you have

set to the NEWEST file in the archive.

Dutester will log any errors that occur to a DUERROR.LOG file for later

evaluation.  The file is created in the directory where DUTESTER.EXE resides.

As you can see, Dutester is one of the finest upload testing programs

available in the BBS world, it has a small filesize (<26K), takes up very

little memory (<31K), and is fast and very easy to use.

INSTALLATION

------------

Be sure the following files are in your Synchronet EXEC directory:

DUTESTER.EXE        KEYFILE .DUT        GIFTEST .EXE

TDCHECK .EXE        PKUNZIP .EXE        PKZIP   .EXE

*LHA     .EXE       *PAK     .EXE        ARJ     .EXE

ZOO     .EXE        SQZ     .EXE        SHROOM  .COM (if doing virus scans)

* PAK.EXE is used for both .PAK and .ARC files.  LHA.EXE is used for

both .LZH and .ICE files.

Optionally, you may place a comment to be placed into ZIP, ARC, PAK, ARJ,

and ZOO type files in your Synchronet TEXT directory, using the name of

COMMENT.TXT.  VERY IMPORTANT: Because of the way ARJ adds comments, your

COMMENT.TXT file can NOT have ANY blank lines at the beginning of the file,

and it MUST have at least ONE blank line at the end of the file!

If you wish to have Dutester check for viruses in programs, you should

also place ONE of the following virus scanning programs into your Synchronet

EXEC directory:

NETSCAN .EXE        SCAN    .EXE        NAV     .EXE        F-PROT  .EXE

VPCSCAN .EXE        HTSCAN  .EXE        TBSCAN  .EXE

IMPORTANT NOTE ON USING VIRUS SCANNERS:  Because of the enormous amount of

memory now required by various virus scanning programs, you MUST run Synchronet

with the following command line if you wish to do virus scans on uploads:

SHROOM -q -t DUTESTER.EXE SBBS

Or, if you are using a batch file to run Synchronet (probably SBBS.BAT), change

it to look like this:

C:

CD \SBBS\NODE1

..\EXEC\SHROOM -q -t DUTESTER.EXE ..\EXEC\SBBS %1 %2 %3 %4 %5

(insert the proper directory names for your BBS)

If you are using Desqview to run Synchronet, your setup should look

something like this:

Program   : C:\SBBS\EXEC\SHROOM.COM

Parameters: -q -t DUTESTER.EXE SBBS

Directory : C:\SBBS\NODE1

(insert the proper directory names for your BBS)

This command line requires the use of SHROOM.COM, this will cause SHROOM

to shrink Synchronet out of memory when the Dutester program is run, allowing

it to do virus scans.  If you do NOT wish to do virus scans, you do NOT need

to use SHROOM, and you MUST run Dutester with the /NOVIRUS switch.

When you've finished all this, you'll need to go into the Synchronet

Configuration utility, select FILE TRANSFERS, then select TESTABLE FILES.

You will need to hit 'INS' to add a new Testable file type, once you do that,

select it, and set it to the following:

File Extension      *

Command Line        %!DUTESTER %f

Working String      Testing with Domain Upload Tester...

Or, to not do virus scans, set the command line to

'%!DUTESTER /NOVIRUS %f'.

If you are using a FOSSIL driver (this will allow users to see the status

of the file testing remotely), then you should set the command line to read

'%!DUTESTER /P=%p /B=%b %f'.  REMEMBER: Most fossil drivers only support baud

rates of up to 38400, so if you have your port locked in SCFG (under Nodes->

Modem Setup) to a rate higher than that, you will need to change that setting.

Also, you will not see what is being displayed to the user (because it

would just end up scrolling off of the screen due to the messages from the

compression and virus programs).

NOTE: A file extension of '*' only works on Synchronet v1b rev 5 or higher, if

you are running an older version of Synchronet, you will need to set

up multiple extensions for the file types you wish to test.  Also note

that if you use any other file testing methods, you no longer need them

because of the versatility of Dutester.

COMMAND LINE OPTIONS

--------------------

The syntax for using Dutester is:

DUTESTER (filename to test) [/options]

The following command line options can be used with Dutester:

/NOVIR   do NOT perform virus scan      /NOLEV   do NOT check multi-level

/NOZIP   do NOT check .ZIP files        /NOARJ   do NOT check .ARJ files

/NOZOO   do NOT check .ZOO files        /NOSQZ   do NOT check .SQZ files

/NOPAK   do NOT check .PAK or .ARC      /NOLHA   do NOT check .LZH or .ICE

/NOTD0   do NOT check .TD0 files        /NOGIF   do NOT check .GIF files

/NOLOG   do NOT log errors              /GIFLT   ALLOW GIFLite'd .GIF files

/SAVE      Save rejected files to DUTESTER.REJ directory

/D[AYS]=x  Delete files older than x days

/P[ORT]=x  Set com port to x (for fossil use)

/B[AUD]=x  Set baud rate to x (for fossil use)

/SCAN    Use McAfee's 'SCAN.EXE' <default>

/NSCAN   Use McAfee's 'NETSCAN.EXE'

/NAV     Use Norton's Anti-Virus 'NAV.EXE'

/VPC     Use Datawatch's Virex 'VPCSCAN.EXE'

/FPROT   Use Frisk Software's 'F-PROT.EXE'

/HT      Use Harry Thijssen's 'HTSCAN.EXE'

/TBSCAN  Use Thunderbyte's 'TBSCAN.EXE'

IMPORTANT: The /NOLEV command line option will ONLY check the FIRST level of

an archive for CRC errors.  It will NOT perform checks of any files

within the archive, and will only perform a virus scan if the file

is an .EXE or .COM file.  (Use along with the /NOVIR option and the

program will be EXTREMELY fast and require very little memory).

NOTE     : Use only ONE virus scanner on the command line, or don't place any

on the command line and Dutester will default to using McAfee's

SCAN.EXE!

The /GIFLT switch ONLY works when you are using Dave Navarro's

GIFTEST.EXE, the program default is to NOT allow GIFLite'd GIF's.

The /SAVE command line option will place rejected files (except

those rejected by date) into a directory called DUTESTER.REJ which

will be created under the directory where DUTESTER.EXE resides.

This option will slow down the operation of the upload tester, also,

there is no notice sent to the BBS to notify the Sysop of programs

in this directory, so if this option is used, the directory should

be checked periodically.

AQUIRING VARIOUS REQUIRED SOFTWARE

----------------------------------

The following programs are SHAREWARE and/or FREEWARE and can be obtained

on various electronic bulletin board systems (including The Beast's Domain

<Domain Entertainment's BBS> @ 714/739-0904 [2400] or 714/521-9439 [USR DS] or

714/739-0669 [ZyXEL 19.2]):

Shroom      Giftest     Tdcheck     Pkzip/Pkunzip       Lharc

Pak         VPCscan     F-Prot      TBscan              Sqz

Arj         Zoo         HTcan

They are not included with Dutester because of the various license requirements

of the copyright holders of those programs.

Norton's Anti Virus is a commercial software, and is available at many

software retail outlets.  The FREEWARE version of Norton Anti Virus

(NAVSCAN.EXE) cannot be used with this software because of the various screen

pauses it contains (this will cause it to hang the BBS until a key is pressed).

NOTE: All programs listed in this documentation file are copyright

by their respective copyright holders.

* DEMONSTRATION VERSION INFORMATION *

The following limitations are placed upon the DEMO version of the program

to encourage registration - all command line features are DISABLED, the DEMO

will ONLY test the first level of archives and will update or remove the

archive file comment (similar to the /NOLEV command).  It will check GIF's and

.TD0 files as well, but it will NOT scan for viruses on any files.

There is also a small delay added when the program is run, just enough of

a delay to be slightly annoying, but not enough to impede the operation of the

bulletin board it is on.

Program History:

Version 1.00 RELEASE

--------------------

Problem found: under certain conditions, 'fopen' would fail to open the

upload tester's error log, I now use 'open' and the problem does not occur.

The new ARJ returns an error level of 9 when processing a file that is

not an ARJ'd file.  This has been implemented for files that may have an

ARJ header but are not ARJ files, rather than exiting with an errorlevel, the

program will continue on (logging the error).

Version 1.00a

-------------

Was looking for the KEYFILE in the current directory, and it needs to be

looked for in the EXEC directory.  Now fixed.

Version 1.00b

-------------

You can now specify a '/DAYS=xxx' on the command line, this will delete

files and archives who's oldest file date is older than xxx days.

Version 1.01

------------

Improved file signature checking for LZH, ZIP, GIF and ZOO format files.

ZOO does not support extracting to a path, it will only extract to the

current directory, so now before extracting a ZOO file, it changes into the

necessary work directory first, and then changes back when complete.

Added 'Origin Filename' to error log.

Added '/SAVE' command line option which will copy rejected files, into a

directory called DUTESTER.REJ under the directory where DUTESTER.EXE resides,

for later evaluation.

Version 1.02

------------

Now DUTESTER makes sure all test directories are empty before extracting

files and performing any tests.

Version 1.10

------------

Domain Upload Tester now supports fossil drivers!

Version 1.10a

-------------

Added support for oddball registration keys.

Fixed a problem in the fossil routines when passed a 0 baudrate.

Version 1.10b

-------------

Screwed up part of the registration checking, fixed.

Found some bugs in the DEMO version, would hang on certain files, and

would delete files in the root directory on certain files, fixed.

Version 1.10c

-------------

Some archive programs would extract programs with the READ ONLY attribute

set, and DUTester would be unable to remove them after testing, DUTester now

changes the file attributes so that it can remove all work files.

Version 1.11

------------

Invalid baudrates would cause Domain Upload Tester to exit with an error

level, they no longer do this.  Also, baud rates above 38,400 are now supported

using the newer X00 (v1.53 and above) fossil driver.

File date testing was not working properly, now fixed.

End of file.